Exp No: 26

TRANSACTION LEVEL MODELING (TLM) USING SYSTEMC

Code:

#include <systemc.h>

#include <tlm>

#include <tlm_utils/simple_initiator_socket.h>

#include <tlm_utils/simple_target_socket.h>

using namespace sc_core;

using namespace tlm;

using namespace tlm_utils;

// Target module

SC_MODULE(Target) {

simple_target_socket<Target> socket;

SC_CTOR(Target) : socket("socket") {

socket.register_b_transport(this, &Target::b_transport);

}

void b_transport(tlm_generic_payload& trans,

sc_time& delay) {

tlm_command command = trans.get_command();

uint64_t address = trans.get_address();

unsigned char* data = trans.get_data_ptr();

delay += sc_time(10, SC_NS);

if (command == TLM_WRITE_COMMAND) {

cout << "Time: " << sc_time_stamp() + delay

<< " -> Target received WRITE"

<< ", Address: " << address

<< ", Data: " << (int)data[0]

<< endl;

} else if (command == TLM_READ_COMMAND) {

data[0] = 100;

cout << "Time: " << sc_time_stamp() + delay

<< " -> Target received READ"

<< ", Address: " << address

<< ", Data sent: " << (int)data[0]

<< endl;

}

trans.set_response_status(TLM_OK_RESPONSE);

}

};

// Initiator module

SC_MODULE(Initiator) {

simple_initiator_socket<Initiator> socket;

SC_CTOR(Initiator) : socket("socket") {

SC_THREAD(process);

}

void process() {

tlm_generic_payload trans;

sc_time delay = SC_ZERO_TIME;

unsigned char data[1];

// WRITE transaction

data[0] = 42;

trans.set_command(TLM_WRITE_COMMAND);

trans.set_address(100);

trans.set_data_ptr(data);

trans.set_data_length(1);

cout << "Time: " << sc_time_stamp()

<< " -> Initiator sending WRITE"

<< endl;

socket->b_transport(trans, delay);

wait(delay);

// READ transaction

delay = SC_ZERO_TIME;

data[0] = 0;

trans.set_command(TLM_READ_COMMAND);

trans.set_address(100);

trans.set_data_ptr(data);

trans.set_data_length(1);

cout << "Time: " << sc_time_stamp()

<< " -> Initiator sending READ"

<< endl;

socket->b_transport(trans, delay);

wait(delay);

cout << "Time: " << sc_time_stamp()

<< " -> Initiator received data: "

<< (int)data[0]

<< endl;

sc_stop();

}

};

// Top-level

int sc_main(int argc, char* argv[]) {

Initiator initiator("Initiator");

Target target("Target");

initiator.socket.bind(target.socket);

sc_start();

return 0;

}

Output: